UNIVERSITA’ POLITECNICA DELLE MARCHE

______________________

FACOLTA’ DI INGEGNERIA

MAXFISH ESP32S3-MATLAB COMMUNICATION

USER MANUAL

V 1.0 from 27 May 2024

Software	3

Hardware	3

Installation	4

Ubuntu 22.04	4

Arduino IDE 2.3.2	4

MATLAB R2023b	4

ROS2 Humble Hawksbill	4

Mosquitto 2.0.18	5

Configuration	6

Pre-Run	6

Run the software	6

Description	7

# Software

- Ubuntu 22.04

- Arduino IDE 2.3.2

- MATLAB R2023b

- ROS2 Humble Hawksbill

- Mosquitto 2.0.18

# Hardware

- M5CoreS3

# Installation

## Ubuntu 22.04

If you already have an Ubuntu 22.04 os, skip this step.

We recommend to install the Windows Subsystem for Linux 2(WSL 2).

Run a Command Prompt as administrator, enable WSL inserting:

dism.exe /online /enable-feature /featurename:Microsoft-Windows-Subsystem-Linux /all /norestart

Enable Virtual Machine Platform:

dism.exe /online /enable-feature /featurename:VirtualMachinePlatform /all /norestart

Restart the computer.

Set WSL 2 as default:

wsl --set-default-version 2

Install Ubuntu 22.04:

wsl --install -d Ubuntu-22.04

Set the username and the password for admin.

You should find Ubuntu in your Windows.

## Arduino IDE 2.3.2

Install Arduino IDE from the link.

In the Arduino IDE menu, select File -> Preferences.

Copy the url into the Additional Board Manager URLs and save:

https://static-cdn.m5stack.com/resource/arduino/package_m5stack_index.json

In the sidebar, select Board Manager, search for “M5Stack”, and click Install.

In Tools select Board: M5CoreS3.

Install “M5CoreS3 library” in Library Manager and “PubSubClient”.

## MATLAB R2023b

Install MATLAB 2023b.

Install ROS Toolbox.

## ROS2 Humble Hawksbill

Follow the official guide to install ROS2 Humble Hawksbill.

## Mosquitto 2.0.18

Install Mosquitto:

sudo apt update -y && sudo apt install mosquitto mosquitto-clients -y

The default config is only bind to localhost, so to allow listeners to bind to external IP address, add in mosquitto.conf file:

allow_anonymous true

listener 1883 0.0.0.0

and run moquitto, specifying the conf file:

mosquitto -c mosquitto.conf -p 1884

# Configuration

## Pre-Run

- Modify in pub_sub_esp32s3.ino the ssid and password with your WiFi network. Modify also mqtt_server with your machine’s IP address.

Note: you should connect both the PC and the ESP32S3 to the same WiFi network.

- In BIDbridge.sh change “source /opt/ros/humble/setup.bash” with the path of your ROS2 setup file.

Change MQTT_HOST with your machine IP address.

Only for WSL:

Assure to disable the Firewall so that the IP address is accessible.

Configure port forwarding on a Windows command prompt:

netsh interface portproxy add v4tov4 listenport=1883 listenaddress=192.168.11.1 connectport=1883 connectaddress=127.0.0.1

## Run the software

- Restart Mosquitto.

- Connect ESP32S3 to the PC and launch the Arduino code.

- Run the BIDbridge.sh file.

- Run the MATLAB code.

# Description

This guide explains how to establish a bidirectional connection between an ESP32S3 and MATLAB, using MQTT and ROS2.

The Arduino pub_sub_esp32s3.ino code initializes the WiFi connection and the MQTT broker, then publishes messages to a MQTT topic (/mqtt1), subscribes another MQTT topic (/mqtt2) and waits for messages.

The BIDbridge.sh script creates a bridge between MQTT topics and ROS2 topics. Specifically, it subscribes to /mqtt1, converts messages and sends them to the /locdata ROS2 topic, while simultaneously subscribing to the ROS2 topic /servocmd and sending messages to /mqtt2.

The MATLAB pub_sub_MATLAB.m script starts a ROS2 node, publishes messages to the ROS2 /servocmd topic, subscribes to the ROS2 /locdata topic and waits for messages.